Quality Assurance Checklist

A checklist is a set of ideas. In testing, checklists come first. They serve as a draft for further activities. Writing and rewriting a checklist is fast and simple. Once the checklist is ready and each item may become the foundation for one or several test cases.

| Project Name | DQ MODULE | Project code | EPM-DQ |
| --- | --- | --- | --- |
| Object Name | Toaster Breville BTA730XL Bit More | QA Engineer | Tamar Tutisani |
| Classification by access | Black box – without possibility to explore internal electrical components, heating elements separately, or disassemble the toaster. |  |  |
| Object description | Overview: A 4-slice toaster with stainless steel construction designed for toasting bagels, thick artisanal bread, and dense pastries. Features wide slots, LED progress monitoring, and multiple toasting functions. Power: 1800 Watts; Voltage: 110–120 Volts. Features: - 4-slice capacity with extra-wide slots - Variable browning control (5 settings) - "Lift & Look" manual function to check toast without interrupting cycle - "A Bit More" button to add extra toasting time - Bagel function for even heat distribution - Defrost function for frozen items - LED indicator showing remaining time - Removable front crumb tray - Brushed stainless steel exterior Model: BTA730XL (Breville Bit More) Web Link Requirements - Power consumption: 1800 Watts - Voltage: 110–120 Volts AC - Dimensions: 8.1"D x 16.1"W x 7.6"H - Material: Brushed stainless steel - Capacity: 4 slices simultaneously - Browning settings: 5 levels - Warranty: 1-year limited product warranty - Weight: ~5–6 lbs (estimated from specs) - Slot width: Extra-wide (fits bagels and artisanal bread up to 1.5" thick) - Crumb tray: Removable, cleanable |  |  |

SMOKE PATH

| Verification area | Test Approach by way of dealing | Would be automated? | Test Type | Expected Result / Comments |
| --- | --- | --- | --- | --- |
| Verify power on/off functionality | Positive | No | Functional | Press power button or push down the lever → heating coils activate and LED indicator illuminates. Press cancel/release → heating stops immediately and toast slots release. No delay or unresponsiveness. |
| Bread slots accept bread without jamming | Positive | No | Functional | Insert 4 slices of standard bread (1 slice per slot) into the toaster without forcing → bread sits evenly in slots and lever presses down smoothly. No bread gets stuck or wedged. |
| Heating elements activate and toast bread | Positive | No | Functional | Set browning to level 3, insert bread, press down lever → heating coils glow (visible through slots), toasting cycle begins, LED shows countdown. After ~2 minutes, bread pops up golden brown, not burnt or pale. |
| Toast ejects completely and safely | Positive | No | Functional | After toasting cycle completes → toast pops up automatically to safe height (at least 1.5 inches above slots) without flying out violently or remaining stuck. User can easily remove toast without burning fingers. |
| Power cord and plug are intact and safe | Positive | No | Functional | Inspect the 1800W power cord for visible damage, fraying, or exposed wires. Plug fits securely into standard 110–120V outlet without loose connection or arcing. No burn marks on prongs. |
| Power cord integrity prevents unsafe operation | Negative | No | Functional | Verify that internal strain reliefs properly secure the 1800W cord against pulling forces. Negative scenario: If excessive tension or a compromised cord casing is simulated, the design prevents internal wire exposure or immediate short-circuit risks. |

CRITICAL PATH

| Verification area | Test Approach by way of dealing | Would be automated? | Test Type | Expected Result / Comments |
| --- | --- | --- | --- | --- |
| Browning control adjusts toast color correctly | Positive | No | Functional | Set browning dial to level 1 (lightest) → toast lightly golden; set to level 5 (darkest) → toast deeply browned. Each level noticeably darker than the previous. Toast color matches setting consistently across multiple trials. |
| "Lift & Look" function allows mid-cycle checking | Positive | No | Functional | During active toasting cycle, manually lift and lower the lever at midpoint → toast is briefly exposed without stopping the cycle. Lever returns to down position smoothly. Toasting resumes and completes normally. |
| "A Bit More" button adds extra toasting time | Positive | No | Functional | Start toasting cycle with browning level 3 → when toast pops up, immediately press "A Bit More" button → toast reinserts and cycles again for ~30–45 additional seconds. Final toast is noticeably darker. |
| Bagel function toasts evenly on cut side | Positive | No | Functional | Insert bagel (cut-side up) into slot, select Bagel function, set browning to level 3 → heating focuses on exposed cut side. After cycle completes, cut side is evenly toasted to golden brown; opposite side is not overcooked or burnt. |
| Defrost function thaws and toasts frozen bread | Positive | No | Functional | Insert frozen bread slice, select Defrost function, set browning to level 3 → toaster extends cycle time to ~3–4 minutes to thaw and toast simultaneously. Bread emerges fully thawed and evenly toasted, not hard or soggy. |
| Crumb tray removes and empties easily | Positive | No | Functional | Locate crumb tray at front bottom of toaster → slide/pull tray out without resistance. Empty accumulated crumbs into dustbin. Reinstall tray → it seats fully and locks in place. Toaster functions normally after reinstall. |
| LED indicator displays remaining time accurately | Positive | No | Functional | Set browning level 3, insert bread, start stopwatch → LED countdown timer begins simultaneously. Compare LED countdown to actual stopwatch time at 1-minute mark and 30-second mark → LED timer matches actual time within ±10 seconds. At cycle completion, LED stops and toast pops up within 2 seconds of timer reaching zero. |

EXTENDED PATH

| Verification area | Test Approach by way of dealing | Would be automated? | Test Type | Expected Result / Comments |
| --- | --- | --- | --- | --- |
| Toaster toasts thick/dense bread without jamming | Positive | No | Functional | Insert thick slice of dense bread (fruit bread, bagel, or artisanal loaf ~1.5" thick) into extra-wide slot, press lever down → bread fits without forcing or jamming. Toasting cycle completes normally. Bread emerges evenly toasted, not jammed or stuck in slots. |
| Toaster handles repeated use without overheating | Positive | No | Reliability | Toast 20 consecutive cycles (4 slices at a time) over 15 minutes at browning level 4 → toaster exterior remains warm but not too hot to touch (under 140°F / 60°C). No smell of burning plastic or component failure. Continues functioning normally. |
| Stainless steel exterior resists fingerprints and stains | Positive | No | Usability / Appearance | After handling toaster during normal use → wipe exterior with dry cloth → no visible fingerprints, water spots, or permanent stains remain. Brushed stainless steel maintains clean appearance. |
| Enclosure prevents foreign moisture ingress | Negative | No | Functional | Verify via design documentation and exterior casing checks that top-housing contours redirect minor accidental spills away from internal live components without creating a short-circuit hazard. |
| Noise level during normal toasting operation | Positive | No | Performance | Place toaster on counter, use sound meter app or decibel meter, measure noise level at 1 meter distance during toasting cycle at browning level 3 → noise should not exceed typical kitchen appliance levels (~75-80 dB). Toaster should not produce high-pitched squealing, grinding, or alarming sounds. |
| Metal objects cannot be safely inserted into slots | Negative | No | Functional | Attempt to insert a metal fork or knife into a toaster slot while toaster is powered off → object should not fit into slots, or if it fits partially, it should not reach heating elements. Do NOT actually insert metal while powered on. Test confirms design prevents accidental electrocution risk if child inserts metallic object. |
| Browning control dial is easy to turn and adjust | Positive | No | Usability | User picks up toaster → locate browning dial on side/front → turn dial through all 5 settings without excessive force or sticking. Dial clicks or moves smoothly between each level. User can adjust browning level intuitively with one hand. No confusion about which direction increases/decreases browning. |
| Button labels and functions are clear and intuitive | Positive | No | Usability | Examine toaster for button labels (Bagel, Defrost, "A Bit More", "Lift & Look", Cancel). Labels are clearly printed/engraved, not faded or illegible. First-time user can identify each button's function without reading manual. Icons or text clearly indicate what each button does. |

Jira Onboarding Task Link:

https://dqdaeau25.atlassian.net/browse/DQ26-102